Data recording operation manual

- Preparing

- Run Matlab GUI, push Refresh button to connect to radar device, push Start button to start.

- Set Transmitter, Receiver, modulation, Bandwidth, Sweep-Time, Sampling-Number etc.

- Recording

- Select Recording Time. for example: 10s

- Push Record button, the display window will stop displaying and start to record raw data.

- Waiting for 10s, a file save window will appear with a default name like PUP20230806T173504.mat,

- which includes the date and time information: 08/06/2023, 17:35:04.

- You can use the default name or assign a new name to it and push save button.

- The GUI will restore its display after file is saved.

- Data loading

- Use “load” command in Matlab command window to read the raw data in. For example:

- “Load PUP20230806T173504”

- After this load command, the raw data matrix and some necessary variables are loaded automatically.

- Use “whos” to observe the data and variables loaded.

- When Transmitter select Tx1, the list is:

- When Transmitter select Tx2, the list is:

- When Transmitter select Tx1.Tx2, the list is:

- The definition of variables are:

- DATE: 	data file saving time, example: 	DATE =”06-Aug-2023 13:29:54”

- LAFH: 	upper frequency, example:		LAFH = 2.5000e+10

- LAFL:  	lower frequency, example: 		LAFL = 2.4000e+10

- LAMS: 	modulation, example: 			LAMS=”sawtooth”

- LANR: 	number of receivers, example:	LANR =1

- LANT:	number of transmitters, example: 	LANT =1

- LARS:	Receiver Name String, example: 	LARS =”Rx1”

- LASN:	Sampling number, example: 		LASN =256

- LAST:	Sweep Time, example:		LAST =5.000E-04

- LATS:	Transmitter Name String, example: 	LATS =”Tx1”

- For the data matrix, the row is sampling number and column is frame, they are all complex numbers.

- Data extraction code examples (ExtractRecordData.m):

load PUP20230806T192612

Tx1Rx1=ComplexDataTx1(1:256, :);

Tx1Rx2=ComplexDataTx1(257:512, :);

figure(1)

plot(abs(Tx1Rx1(:, 16)))

figure(2)

plot(abs(Tx1Rx2(:, 200)))